Technical Specifications for the software part of the system (on the plant side)

General requirements:

- The software part of the overall system should have a modular architecture (m.b. microservice architecture);

- The system should be distributed in it’s essence as it has spatially separated parts – part for data collection and monitoring (on the plant side), part for remote system control and model training and updating (on our side) and remote data storage part (cloud etc);

- The implementation of the system should not have OS dependencies;

- In the case of the plant does not work around-the-clock the part on the factory side should work on a certain schedule for a resources saving. The schedule of our system should be defined by the plant work schedule;

- Operating regime (schedule) of our system should be defined through the configuration file (config.ini file);

Functionality:

Configuration Reading Module is responsible for reading configuration settings from config.ini file and transmitting them to Controller Module. Configuration file should contain at least working schedule of the plant: start working time, stop working time; collected data transmission time to remote storage; subscription conditions; m.b. some other additional parameters.

Controller Module is responsible for managing the scheduling process of work of other modules and working process of overall system. It defines: when to start process of data collection; when to stop data collection; when to transfer all collected in local file system data to remote storage; to deliver results of the work of our system to customer or not (subscription conditions compliance handling); to manage the process of sending warning etc messages; to manage the process of deleting collected files from local storage; to manage the state of the system; m.b. something else. I suppose in the future if our system show good results this module should have possibility to connect to remote service to receive necessary information (for example to manage subscription conditions compliance, etc)

Data Collection Module is responsible for data collection in it’s buffers and for passing data to Data Storage Module. The moment of passing collected data from Data Collection Module to Data Storage Module is determined by the module itself.

Data Storage Module is responsible for saving received from Data Collection Module data into local file system and for transferring data from local file system to remote storage (by the signal from Controller Module).

Logging Module should to log service information on local storage for example message about internet connection loss or about problems during data collection or transmission etc. The work of the module is controlled by Controller Module.

Messenger Module should to send service messages to remote system/”subscriber” (if it’s possible in the case of existence of internet connection), for example about serviceability of the overall system.

System Validation Module is responsible for definition of the working process of the overall system. First of all it must validate the state of sensors by analyzing received signals (for example if any of sensors has fallen off etc) and sending service messages (“VALID STATE” or “INVALID STATE”) to Controller Module which in the case of some malfunctions should to made Logger Module to log this event and Messenger Module send message about this event to remote “subscriber” and to change working mode of the system into INVALID STATE and stop data collection and transmission etc. If Controller Module receive “VALID STATE” signal from the System Validation Module then it should to set up the state of the system into VALID STATE mode (if it was not in the state before).

- Approximately 30 minutes before the official start of the working day on the plant our system should start recording of input data streams receiving from sensors after analog-to-digital conversion. Initially data have to be collected into specially intended memory buffers. Possibly it should be allocated two buffers for each data channel to prevent data loss in the moment of saving collected data into file system (may be it is overly dependently on drivers and used frameworks because they have their internal buffers). Buffers size depends on sample rate, number of bits and time frame size (for the moment it does not matter). In the case of working around-the-clock regime process of data collection and recording should be continuous.

- Approximately 30-60 minutes pass after stop working day on the plant data recording process should be stopped (in the case of the plant does not work around-the-clock);

- One time in a predefined period of time (30-60 min) data collected in buffers should be saved on file system as separated files (approximate file size will be 75 to 150 Mb) Data Collection Module pass collected data to Data Storage Module which saves data to local file system;

- On the predefined schedule (from configuration file) Controller Module sends signal to Data Storage Module to send portion of the collected data to remote storage. Data Storage Module needs to receive confirmation of successful data transfer from remote storage. In the case of data transmission failure Logger Module should write down message about this event and Messenger Module should try to send service message to “subscriber” remote service. After the pause the module should retry to transfer the same data to remote storage until it succeed or it is stopped by Controller Module.

TO BE CONTINUED

System Validation

Module

Messenger

Module

Configuration Reading Module

Logging

Module

Controller Module

Data Processing Module

Data Collection Module

Data Storage Module